SPROCKET DESIGN REPORT

I.Introduction

- The 18-tooth and 30-tooth sprockets were designed to provide efficient power transmission and the required gear reduction for the drivetrain. The design focuses on strength, lightweight construction, proper chain engagement, and durability while ensuring smooth operation and minimal power loss.

II.Rulebook Constraints

- Sprockets are securely mounted to their respective shafts and designed to withstand all anticipated torque loads.

- All sprockets are enclosed within a rule-compliant scatter shield.

- Adequate chain wrap and alignment are maintained to prevent derailment during dynamic events.

- The design ensures safe operation throughout acceleration, endurance, and autocross events.

III.Dimensional Parameters

18-toothed Sprocket

| Parameter | values |
| --- | --- |
| No.of teeth | 18 |
| Chain Pitch | 12.7mm |
| Pitch Diameter | 36.82 |
| Root Diameter | 32 |
| Outside Diameter | 39.83 |
| Torque Capacity | 86Nm |

30-toothed Sprocket

| Parameter | values |
| --- | --- |
| No.of teeth | 30 |
| Chain Pitch | 12.7mm |
| Pitch Diameter | 60.75mm |
| Root Diameter | 56.72mm |
| Outside Diameter | 64.2mm |
| Torque Capacity | 86Nm |

IV.Calculations

For 18-teeth sprocket

Given:

Number of Teeth, N = 18

Outer Diameter, OD = 39.82 mm

Pitch Diameter, PD = 36.30 mm

Root Diameter, RD = 32.00 mm

Material = 6082-T6 Aluminium Alloy

Yield Strength, σy = 260 MPa

Applied Torque, T = 51.6 N·m

Sprocket Thickness, t = 7.2 mm

1.Pitch Radius

r = PD / 2

r = 36.30 / 2

r = 18.15 mm

r = 0.01815 m

2.Tangential Force Acting on Sprocket

Ft = T / r

Ft = 51.6 / 0.01815

Ft = 2842.98 N

Ft = 2.84 kN

3.Load per Tooth

Assuming three teeth share the load:

Ftooth = Ft / 3

Ftooth = 2842.98 / 3

Ftooth = 947.66 N

4.Circular Pitch

p = (π × PD) / N

p = (3.1416 × 36.30) / 18

p = 6.34 mm

5.Tooth Contact Area

A = t × p

A = 7.2 × 6.34

A = 45.65 mm²

6.Bearing Stress on Tooth

σb = Ftooth / A

σb = 947.66 / 45.65

σb = 20.76 MPa

7.Factor of Safety

FoS = σy / σb

FoS = 260 / 20.76

FoS = 12.52

RESULTS

Pitch Radius = 18.15 mm

Tangential Force = 2842.98 N

Load per Tooth = 947.66 N

Circular Pitch = 6.34 mm

Contact Area = 45.65 mm²

Bearing Stress = 20.76 MPa

Yield Strength = 260 MPa

Factor of Safety = 12.52

For 30-teeth sprocket

Given:

Number of Teeth, N = 30

Chain Pitch = 12.7 mm

Pitch Diameter, PD = 60.75 mm

Root Diameter, RD = 56.72 mm

Outside Diameter, OD = 64.20 mm

Material = 6082-T6 Aluminium Alloy

Yield Strength, σy = 260 MPa

Applied Torque, T = 86 N·m

Sprocket Thickness, t = 7.2 mm

1.Pitch Radius

r = PD / 2

r = 60.75 / 2

r = 30.375 mm

r = 0.030375 m

2.Tangential Force Acting on Sprocket

Ft = T / r

Ft = 86 / 0.030375

Ft = 2831.28 N

Ft = 2.83 kN

3.Load per Tooth

Assuming three teeth share the load:

Ftooth = Ft / 3

Ftooth = 2831.28 / 3

Ftooth = 943.76 N

4.Circular Pitch

p = (π × PD) / N

p = (3.1416 × 60.75) / 30

p = 6.36 mm

5.Tooth Contact Area

A = t × p

A = 7.2 × 6.36

A = 45.79 mm²

6.Bearing Stress on Tooth

σb = Ftooth / A

σb = 943.76 / 45.79

σb = 20.61 MPa

7.Factor of Safety

FoS = σy / σb

FoS = 260 / 20.61

FoS = 12.62

RESULTS

Pitch Radius = 30.375 mm

Tangential Force = 2831.28 N

Load per Tooth = 943.76 N

Circular Pitch = 6.36 mm

Contact Area = 45.79 mm²

Bearing Stress = 20.61 MPa

Yield Strength = 260 MPa

Factor of Safety = 12.62